Duties and Responsibilities of an Engineer

General Duties

- To obey all lawful commands of the Captain and Mate.

- To perform duties in a professional and seaman-like manner.

- To assist the Captain

- To maintain the vessel in a constant and proper state of repair and operational readiness.

- To take charge and be responsible for the good condition and efficient working of the entire machinery plant of the vessel.

Specific Duties

- To drain all air tanks as required

- To grease all machinery as required.

- To check battery water level as required.

- To take oil samples monthly or as required.

- To clean strainers.

- To change oil as required on all equipment at regular intervals determined by the Port Engineer.

- To check for water in fuel tanks daily and when fueling.

- To test and run all pumps as required.

- To check for and repair any leaks as needed.

- To keep all loose wiring and cabinets secure on electrical systems.

- To check all fire and safety equipment and let the Captain and Port Engineer know if any of the equipment is defective or out of date.

- To monitor all temperatures and pressures while standing engine room watch.

- To keep engine room space clean and free of trash at all times.

- To maintain and be aware of spare parts inventory onboard.

- To keep track of all supplies ordered and received.

- To report to the Captain and office with breakdowns and/or problems.

- To keep an accurate daily engine room log.  List all repairs, maintenance and duties performed each day.

- Sign log book each day.

- To maintain personal hygiene (i.e. bathe regularly, wear clean clothes) and keep living areas in a clean and sanitary condition.

- When tying up the vessel, follow these steps:

  - Check packing glands – tighten if necessary

  - Fuel sight tubes shut off

  - Close Sea chest

  - Turn off engine alarms.

  - Covers on stacks.

  - Running on the _________ tank.

  - Check bilge level.

  - Check heat on vessel.

  - Log actions and observations.

  - Plug in shore power.

  - Make sure cable has enough slack for tidal range.

  - If cable will not reach, extension cords must be run to the closest boat to power the heat and refrigerator.

  - Make sure lines have enough slack for tidal range.

  - Make sure that the vessel is secured – all hatches dogged and locked.