Pollution Prevention

- Fuel oil and bulk lubricating oil containment (33 CFR 155.320)

- Oily Mixtures (Bilge slop) retention (33 CFR 155.330, 155.350, 155.360, 155.370)

- c) Bilge pumps or other dewatering capability:   All towing vessels must have either an installed bilge pump or another method for emergency dewatering, such as a portable pump with hoses.

- d) Oil transfer hoses (33 CFR 154.500)

- e) Oily water separator equipment (where applicable) (33 CFR 155.350, 155.360, 155.370, 155.380)

- f) Placard prohibiting discharge of oil (33 CFR 155.450)

- g) Garbage (MARPOL) placard (33 CFR 151.59)

- h) Garbage disposal requirements/procedures/documentation (including Waste Management Plan where applicable) (33 CFR 151.57, 33 CFR 151.63; 151.66; 151.67; and 46 CFR 25.50-1)

- Marine Sanitation Devices (MSD) (33 CFR Part 159)

- j) Oil transfer procedures (33 CFR 155.720, 33CFR155.750)

- k) Declaration of Inspection (33 CFR 156.150, 33 CFR 156.120)

- Pumping systems for oily mixtures (33 CFR 155.410, and 155.420)

- m) Spill kit:  During fueling operations, each towing vessel must have ready for immediate use equipment and supplies to clean up and remove an on-deck oil spill of at least one barrel. The equipment and supplies must include sorbents; non-sparking hand scoops, shovels, and buckets; containers suitable for holding recovered waste; emulsifiers for deck cleaning; and protective clothing.

- n) Closable scuppers or other containment method:  Each towing vessel must be capable of preventing spills to the deck during oil or fuel transfers from reaching the water, either by 1) pre-closing off the scuppers/freeing ports, if the vessel is so equipped, or 2) pre-deploying absorbent booming/pads on the deck around vents and fills.