STANDARD OPERATING PROCEDURES

FUEL TRANSFER

- Sound tanks.

  - “T” handle to open tank top is in engine room next to after engine room access ladder – at shoulder level.

  - Use measuring stick on starboard tank that is stored next to after engine room access ladder.

  - Port side measuring stick is in engine room against port fuel tank.

  - Each stick is calibrated to 450 gallons

  - If measuring just the starboard side, add twenty (20) gallons to total.  Port side usually has 20 gallons more than the starboard side due to slight list caused by generator.

  - Always have oil absorbent rags on deck when opening a tank top.

- Record fuel level in engine room log.

  - Example:  350 gallons starboard side, 720 gallons total

- Clean measuring stick off and put back in engine room.

- Fly red flag while fueling.

- Fill tank through starboard tank – same place where you sound tank.

- A fast fill is less likely to foam.

- Always maintain a watch while fueling.

  - Place oil absorbent rags around tank opening with an extra layer towards the outboard side and stern.

  - Never walk away from open tank – whether fueling or not.

- As fuel level nears top, slow down fuel transfer rate.  A quick stop will cause fuel to “surge” in tank and may overflow.

- If filling completely, take as much as possible.  The top of the tank is the deck.

- When finished, have attendant shut off valve at pump, then, drain hose.

- Wipe hose end off with oil absorbent rag and hold end with rag (covering outlet) until attendant takes hose from you.

- Cover tank, but don’t screw down.  Wait several minutes for fuel to equalize in both tanks through crossover pipe, then open and sound tank again.  Secure cover.

- Clean up area around tank cover.  Dispose of any soiled rags in engine room’s metal waste bucket.

- Record fuel level in engine room log.

- Record fuel amount taken, as well as price, in engine room log.

- If you are unsure of anything, stop and ask a crew member or call the office.